Digital assignment

Due date 16-5-2024 6:00PM

Q1) [8 points] Using 3 bit comparators and any required digital components, design a circuit that compares three numbers. Each number is 3bits. The largest number will be displayed on 7 segment display. (use block diagrams of the comparators)

______________________________________________________________________

Q2) [8 points] Using JK flip flops, design a counter that counts 0-2-4-6 and back to zero if the input is 1. And it will count 0-6-4-2 and back to 0 if the input is 0. The count will be displayed on 7 segment display.

______________________________________________________________________

Q3)  [9 points] The bank uses a queue system to ensure the privacy of its customers.

Each customer gets a number and waits for his/her turn.

Each teller (موظف البنك) announces the customer number which will be serviced using two seven-segment displays: one to show the customer‘s number and the other to show the teller's desk number.

Each teller has

1- a push button to call the next customer

2- A seven-segment display to display the customer number.

3- A unique desk number

The system has

1- A counter

2- push buttons

3- One main seven-segment display to show the customer’s number

4- One main seven-segment display to show the teller’s number

5- One seven-segment display for each teller to show the customer’s number.

When a teller presses the push button

The counter increases by one

The content of the counter is displayed on the teller 7-segment display.

The content of the counter is displayed on the main 7-segment display

The teller number who pressed the button is displayed on the main 7-segment display associated with the teller desk number

Suppose the bank has 3 tellers, the customers’ numbers are from 0-9

you have 5 Seven-segment displays and one counter.

- Use a block diagram in your design

- Show the gate-level design for the counter.